LAYOUT

Outside World
(master)

Slave Device
(slave)

Pairing Module
(master to Slave Device)

(Slave to Outside World)

function

2 wires TTL

Definition of devices:

Slave Device: A device that supports Modbus RS232 communication protocol as a Modbus Slave and contains the code needs to communicate and pair with a pairing Module. Example: an energy meter.

Pairing Module: A Module that acts as a bridge between the Modbus Slave (using Modbus RS232 for communication) and the Outside World (other devices inside the Modbus Network) with and RS232/RS485 Interface.

Each Slave Device is designed to match a unique Pairing Module.

Outside Word: Any device in the Modbus network. These devices will try to consult the values stored inside the Slave Device Modbus Map.

Resume:

The Pairing module acts as a bridge between the Slave Device and the Outside world. This behavior will happen only after the modules are “Paired” (meaning that the module and the device have acknowledged communication and compatibility between each other). Compatibility is ensured by the following method. During production a unique UUID code (12 bytes length) is stored inside each Pairing Module and Slave Device FLASH memory. Meaning that a Slave Device and its corresponding Pairing Module will have the same unique UUID code. As will be explained, the matching of the UUID code between the Slave Device and Pairing Module will enable the Pairing Module to act as a bridge between the Slave Device and the Outside World.

After the pairing is complete, the Pairing Module will set its Modbus Address as the Address that the Slave Device sends to the module and acts as Modbus Slave from the point of view of the Outside world  (Exterior Modbus Network). This means that any Modbus packet sent to the Slave Device configured address will be in fact received by the Pairing Module and resend to the Slave Device. Then the Slave Device will process this packet and will send the response that Pairing module has to send to the Outside World.  Also, the Pairing module will regularly check that the Slave Device is still working and in communication with the Pairing Module by sending a Keep Alive message to the Slave Device and wait for its acknowledgement.

Power ON states

Power ON

Pairing & Exchange Configuration

Data Exchange and check new configuration

Description of the Pairing procedure and behavior of devices.

| Step | Slave Device (such as EM540) | Pairing Module (S1, O1) |
| --- | --- | --- |
| 1 | Slave device Modbus sets its parameters for communication with the module: Modbus RTU slave address: 0x5A. Baud Rate: 115200. Data: 8 bits. Parity: No. Stopbit:1. | Sends “Pairing Message” to Modbus address 0x5A. |
| 2 | The Slave device will be waiting to receive a Modbus message from the Module (ignoring any other messages), | Keeps Sending “Pairing Message” until the module responds. |
| 3 | -Receives “Pairing Message” from the Pairing Module. -Verifies that the UUID (12 bytes length) code inside the pairing message received matches the code stored inside the Salve Device FLASH memory. -if the code does NOT match, send a match error to notify the module. -If the code matches, get the number of communication channel. -Set the slave address (slave ID) of the Slave device as the communication channel value. -Send the response to the Pairing Module. |  |
| 4 |  | The module receives the pairing confirmation message with the communication channel. From now on, the module uses the slave address (slave ID) to communicate on this channel |
| 5 |  | The module sent its revision to Slave Device with a modbus write operation to register 0x2200 0xXX ¦0x06¦0x22¦0x00¦0xYY¦0xZZ¦0xcrc¦0xcrc 0xYYZZ = module revision-> 0x1103 = 1.1.3 |
| 6 |  | The module reads its configuration from the device: - S1: read 5 register from 0x2000 see EM540 communication protocol for more details: - 0x2000 = external address - 0x2001 = baudrate - 0x2002 = parity - 0x2003 = stop bit - 0x2004 = delay on response - O1: read 16 register from 0x1012 : - 0x1012 = Ton Time 0 = 30ms 1 = 100ms - …all the registers in the middle are not used for this functionality… - 0x101E = digital output type 0 = Disabled 1 = Pulse kWh+ 2 = Pulse kWh- 3 = Alarm - 0x101F = NormallyOpenClose 0= normally open 1 = normally close - 0x1020-0x1021 = Pulse Weight 0 = 10000 1 = 1000 2 = 100 3 = 10 4 = 2 5 = 1 |
| 7 |  | The module cleans the configuration bit. 0xXX ¦0x06¦0x50¦0x19¦0x00¦0x00¦0xcrc¦0xcrc The configuration bit is a particular bit on register 0x5019 that is put to 0x0001 by the “Slave Device” if the current configuration is changed. |

Message structure and other information.

Every “pairing module” has a specific “code” that identify the fuctionality.

Code values:

- 1000 : Serial module S1

- 2000 : Mbus module M1

- 3000 : Output module O1

This “code” is implemented also into the Slave Device in this way:

- The ”code” is used in the UI (display and keyboard) that exposes differents configurations pages and info pages

- Configured during the production step

- During the “pairing” step the “measure module” rejects all modules that do not have the same “measure module” “code”

Every “pairing  module” must have also a UUID code (for S1, M1 and O1 module the uuid is the stm32 uid of microcontroller)

The UUID code is used during the production process to match the “pairing  module” to “Slave device”, this matching is valid for whole device life.

If a module is replaced with another one (also with the same “code”) the “measure module” reject the module connected during the “pairing” step.

S1, M1 and O1 module have a digital input. This input information is sent to the “Measure Module” in a special fast way during “Data excange” step.

Pairing packets: the first step creates a “communication” channel between the module (modbus master side) and measure module (modbus slave side)

The pairing packet exchanged is modbus like:

Pairing Module (master):

| 0x5A | 0x10 | 0xF1 | 0xA5 | 0x00 | 0x07 | 0x0E | 0xXX | 0xXX | 0xXX | 0xXX | 0xXX | 0xXX | 0xXX | 0xXX | 0xXX | 0xXX | 0xXX | 0xXX | 0x03 | 0xE8 | 0xCR | 0xCR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Modbus write 0x10

with slave address 0x5A @ reg 0xF1A5, 7 registers (0x0007)

UUID code (12 bytes)

Code:
1000 = modbus module

Modbus CRC

Measure Module (slave device):

Response OK

| 0x5A | 0x10 | 0xF1 | 0xA5 | 0xC1 | 0xXX | 0xCR | 0xCR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Modbus CRC

Modbus write 0x10 responce like

Pairing procedure completed

Modbus address of measure module from now : “MB ADDRESS”

Response NOK

| 0x5A | 0x90 | 0x04 | 0xCR | 0xCR |
| --- | --- | --- | --- | --- |

Data exchange: S1 module

The communication between S1 module and “Device Slave” module has triggered by:

- An external communication: a packet has received from RS485 line.

- An internal timeout: no RS485 communication in 100ms

External communication: from the received packet the modbus address is substitute with “MB ADDRESS” and the input status is write on “MB ADDRESS” in this way.

After this the CRC is recalculated and the message is sent to “Device Slave”

An internal timeout: no RS485 communication in 100ms, so an internal message is populated. In this case there are 2 possible messages:

- uint8_t const c_templatePingNoTxRxBuff[] = {0x5A, 0x03, 0x50, 0x19, 0x00, 0x01, 0xFF, 0xFF} ;

- uint8_t const c_templatePingShowRxBuff[] = {0x5A, 0x08, 0x50, 0x19, 0x00, 0x01, 0xFF, 0xFF} ;

The first one does not cause a Tx and Rx blink icon on display (pure internal communication)

The second one causes a Rx blink icon on display (a messages is received on module but it is not for this device)

Before the relevant message has sent the modbus address is substitute with “MB ADDRESS” and the input status is write on “MB ADDRESS” in this way.

After this the CRC is recalculated and the message is sent to “Device Slave”.

Common:

On both data exchange the response from “Device Slave” contains in the modbus address there’s the “change configuration” flag.

If the changeConf flag is set to 1, the device reads its configuration (such as during the pairing) and after clean the configuration bit:

0xXX|0x06|0x50|0x19|0x00|0x00|0xcrc|0xcrc

Data Exchange: O1 module

In this case the packet depends on the configured function (configuration read @ register 0x101E) :

MB_OUTPUT_DISABLED:			 {0x5A, 0x03, 0x00, 0x0B, 0x00, 0x01, 0xFF, 0xFF} ;

MB_OUTPUT_CNT_KWH_IMPORTED: 		{0x5A, 0x03, 0x05, 0x00, 0x00, 0x04, 0xFF, 0xFF}

MB_OUTPUT_CNT_KWH_EXPORTED: 		{0x5A, 0x03, 0x05, 0x1C, 0x00, 0x04, 0xFF, 0xFF} ;

MB_OTHER: 				{0x5A, 0x03, 0x03, 0x05, 0x00, 0x01, 0xFF, 0xFF} ;

the modbus address is substitute with“MB ADDRESS” and the input status is write on “MB ADDRESS” in this way.

After this the crc is recalculated and the message is sent to “Device Slave”

On data exchange the response from “Device Slave” into the modbus address there’s the “change configuration” flag.

If the “chage conf flag is set to 1 the device read its configuration (such as during the pairing) and after clean the configuration bit :

0xXX |0x06|0x50|0x19|0x00|0x00|0xcrc|0xcrc

FAQ:

What happens if the pairing has not been done but the Pairing Module receives a message that matches the current Modbus address of the Slave Device.

Answer: The Pairing Module will receive the packets, but it will respond with a Modbus error to the device from the Outside World that sends the Modbus message and will not send anything to the Slave Device because the Pairing Process has not been done. This means that the Slave Device will only receive continuous messages from the Pairing Module that will attempt to complete the Pairing Procedure.

What happens if I change my slave device’s baud rate or other communication parameters?

The Slave Device is programmed to use the same communication parameters as the Pairing Module when boot up. After the pairing is done, the slave device will pass the Modbus communication parameters (like baud rate, stop bits, ect.) stored in the device memory to the Pairing Module, which will use these parameters to configure its modbus communication with the Outside World.  This also means that the Slave device must re-start in order to do again the pairing procedure and pass the Modbus communication parameters stored in the internal memory (that, in this case, were modified by the user by changing the baud rate or any other parameter) to the Pairing Module.